Attribute Control Chart Examples     Fall 2011

Thirteen samples of size n=100 each were collected. The number of defective items for each sample are shown below.

Sample	Number of

#	Defectives

1	5

2	4

3	3

4	3

5	2

6	4

7	4

8	4

9	2

10	5

11	2

12	6

13	3

Use Minitab to build a P chart and evaluate the process control.

Results:

The y-axis of the P chart is the proportion defective for each sample.

The process does not exhibit any out-of-control characteristics.

Given the following data on defects and sample size, use Minitab to build a C chart and evaluate the process control.

Example

DataC	SizeC

2	50

1	50

3	50

4	50

2	50

4	50

6	50

4	50

2	50

5	50

2	50

5	50

3	50

Note that there were 13 samples of size n=50 taken.

Note that Minitab does not ask for the sample size. As is the case for P charts, you are given the option of performing tests for special causes.

Results:

Sum of ExampleData_C = 43

Mean of ExampleData_C = 43/13  =  3.30769  = cbar

Square root of cbar   = 1.81871

UCL = cbar+ 3 sqrt cbar   =  8.76381

Note that the sample size information is not used for calculating C bar or the control limits. C bar is calculated as the sum of the defects divided by the samples taken.  In this example C bar = 31/13.

Note the label on the y-axis of the Minitab C chart. The Sample Count is the number of defects in each of the thirteen samples.

Given the following data on defects and sample size, use Minitab to build a U chart and evaluate the process control. The U chart measures defects per unit and is used when the number of units inspected varies from one sample to another.

DataU	SizeU

5	50

4	60

3	40

5	40

5	50

5	60

5	30

4	30

3	30

1	30

1	30

6	50

3	50

2	50

5	60

2	60

The centerline of the U chart is the overall U bar (total defects / total items sampled).

Note the label on the y-axis of the U charts (defects per unit).  Also note that the two charts appear to be alike, even though one tested for all special causes and the other tested for none.